# SIMATS ENGINEERING

## ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | GOKUL RAJ | Reg.No.: | 192521494 |
| Department: | IT | Date: | 27 08 2026 |

### ANNEXURE A

COMPARATIVE ANALYSIS

- Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

Comparison of L1, L2, and L3 Cache

Cache memory is a small, high-speed memory located close to or inside the processor. It stores frequently accessed instructions and data so that the CPU does not always need to access slower main memory.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Closest to CPU core | Close to/inside CPU core | Usually shared among cores |
| Typical Size | Few KB to ~128 KB per core | Hundreds of KB to a few MB | Several MB to tens of MB |
| Latency | Lowest, about 1–4 CPU cycles | Higher, about 4–15 cycles | Higher, about 20–50+ cycles |
| Bandwidth | Very high | High | Lower than L1/L2 |
| Throughput | Highest | Very high | High |
| Cost per bit | Highest | High | Lower |
| Purpose | Immediate CPU data/instructions | Secondary fast storage | Large shared cache |
| Access frequency | Very high | High | Moderate |

L1 Cache

L1 is the fastest cache and is normally divided into instruction cache (L1-I) and data cache (L1-D). Because it is very small, it can be accessed extremely quickly. A successful L1 access, called an L1 hit, allows the processor to continue with minimal delay.

L2 Cache

L2 is larger than L1 but slower. If the required data is not found in L1, the processor checks L2. L2 provides a balance between capacity and speed.

L3 Cache

L3 is larger and slower than L1 and L2. In modern multicore processors, L3 is commonly shared among several or all CPU cores. It reduces the number of expensive accesses to DRAM.

Impact on Processor Performance

The effectiveness of cache is strongly related to the cache hit rate.

- A high L1 hit rate gives extremely low access latency.

- L2 catches many accesses that miss in L1.

- L3 reduces accesses to relatively slow main memory.

- Better cache locality increases processor throughput.

- A cache miss causes additional latency because data must be fetched from the next memory level.

Therefore, a processor with effective cache organization can execute instructions much faster than one that frequently accesses DRAM.

Conclusion: L1 provides the lowest latency and highest bandwidth, L2 provides greater capacity with slightly higher latency, and L3 provides large shared capacity while still being much faster than main memory.

- Differentiate	SRAM	and	DRAM	by	evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

SRAM vs DRAM

SRAM (Static Random Access Memory) and DRAM (Dynamic Random Access Memory) are both volatile semiconductor memories, but they have different structures and performance characteristics.

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Storage element | Flip-flop, typically 6 transistors | Capacitor + transistor |
| Latency | Very low | Higher |
| Bandwidth | Very high | High |
| Throughput | Very high for small, frequent accesses | High for large sequential transfers |
| Refresh required | No | Yes |
| Density | Low | High |
| Cost per bit | High | Low |
| Power per bit | Relatively high area/cost | Lower area per bit, but refresh consumes energy |
| Typical use | CPU caches | Main memory |

SRAM in Cache Design

SRAM is normally used for L1, L2, and L3 caches because it provides very fast access. Its stored data remains available as long as power is supplied, without periodic refresh.

The main advantage is low latency. This is important because CPUs operate at extremely high clock frequencies and cannot afford frequent long waits for memory.

However, SRAM requires more transistors per bit, making it relatively large and expensive. Therefore, building a very large SRAM cache would significantly increase processor size and cost.

DRAM in Main Memory

DRAM stores each bit using a transistor and capacitor. The capacitor gradually loses its charge, so DRAM must periodically be refreshed.

DRAM has higher latency than SRAM, but it offers much greater density at a lower cost per bit. This makes it suitable for large-capacity main memory.

Modern DRAM systems also use wide memory buses, multiple channels, and burst transfers to achieve high bandwidth.

Latency, Bandwidth and Throughput

Latency: SRAM is significantly faster because its storage cells can be accessed without the sensing and refreshing operations associated with DRAM.

Bandwidth: SRAM can provide extremely high bandwidth when placed close to the processor. DRAM can also provide high aggregate bandwidth through multiple channels and high-speed interfaces, but individual access latency remains higher.

Throughput: SRAM is excellent for small, random, repeated accesses typical of CPU caches. DRAM achieves good throughput when transferring large blocks of data.

Conclusion

SRAM is preferred for cache memory because speed is more important than capacity. DRAM is preferred for main memory because capacity and cost efficiency are more important.

The basic hierarchy is:

CPU → SRAM Cache → DRAM Main Memory → Secondary Storage

This combination provides a good balance between speed, capacity, and cost.

- Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

- Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

Virtual Memory, Paging, and Cache Memory

Virtual memory and cache memory both improve the usability and performance of computer systems, but they solve different problems.

| Feature | Virtual Memory | Paging | Cache Memory |
| --- | --- | --- | --- |
| Main purpose | Gives processes a large virtual address space | Manages virtual-to-physical memory mapping | Reduces CPU memory access time |
| Storage involved | RAM + secondary storage | RAM + secondary storage | SRAM cache + RAM |
| Typical latency | High when disk/SSD access occurs | High on a page fault | Very low on a cache hit |
| Unit of transfer | Pages | Fixed-size pages | Cache blocks/lines |
| Main benefit | Memory abstraction and isolation | Efficient memory allocation | Faster data access |
| Performance issue | Page faults | Page faults and translation overhead | Cache misses |

Virtual Memory

Virtual memory allows a process to use a virtual address space that can be larger than the available physical RAM. The operating system maps virtual addresses to physical memory.

When required data is already in RAM, access can proceed normally after address translation. However, if the required page is not in RAM, a page fault occurs.

The operating system must retrieve the page from secondary storage, which can introduce extremely large delays compared with RAM access.

Paging

Paging divides virtual and physical memory into fixed-size units called pages and page frames.

A page table records the mapping between virtual pages and physical frames. A Translation Lookaside Buffer (TLB) is used to cache recent address translations.

A TLB hit can make address translation fast. A TLB miss requires additional page-table accesses, increasing latency.

If the page itself is absent from physical memory, a page fault occurs. This is much more expensive than either a TLB miss or a cache miss.

Cache Memory

Cache memory stores recently or frequently used data close to the processor.

When the CPU requests data:

- The processor checks L1 cache.

- If it misses, it checks L2.

- If L2 misses, L3 may be checked.

- Eventually, main memory may be accessed.

A cache hit has very low latency, while a cache miss increases access time.

Effect on Throughput

Cache memory primarily improves CPU throughput by reducing the average memory access time.

Virtual memory primarily provides capacity, protection, isolation, and address-space flexibility, rather than making memory faster.

Paging improves memory utilization by allowing only the currently needed pages to occupy physical memory. However, excessive page faults can cause thrashing, where the system spends much of its time moving pages instead of executing programs.

Overall Relationship

The mechanisms work together:

CPU → Cache → TLB → RAM → Virtual Memory/Storage

Cache reduces the time needed to access frequently used data, while paging and virtual memory manage how programs use physical memory.

Conclusion: Cache memory is primarily a performance mechanism, whereas virtual memory and paging are primarily memory-management mechanisms. Cache hits can take only a few CPU cycles, while page faults can be many orders of magnitude slower.

- Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

MRAM vs RRAM

MRAM (Magnetoresistive Random Access Memory) and RRAM/ReRAM (Resistive Random Access Memory) are emerging non-volatile memory technologies. Both aim to provide fast, persistent memory while reducing some limitations of conventional memory technologies.

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Storage principle | Magnetic state | Resistance state |
| Volatility | Non-volatile | Non-volatile |
| Access time | Very fast | Very fast to moderate, depending on implementation |
| Read performance | High | High |
| Write performance | High | Potentially high |
| Throughput | High | High potential |
| Energy efficiency | Good | Potentially very good |
| Endurance | Very high | Generally high, but device-dependent |
| Data retention | Excellent | Good to excellent, device-dependent |
| Density | Good | Potentially very high |
| Technology maturity | Relatively mature | More emerging |
| Potential applications | Embedded memory, cache, persistent memory | High-density memory, neuromorphic and storage-class applications |

MRAM

MRAM stores information using magnetic states rather than electrical charge.

A common form is STT-MRAM (Spin-Transfer Torque MRAM), while newer approaches include SOT-MRAM (Spin-Orbit Torque MRAM).

MRAM has several important advantages:

- Non-volatility

- Fast read access

- Very high endurance

- Good energy efficiency

- No refresh operation

- Good resistance to data loss when power is removed

Because MRAM does not require periodic refresh like DRAM, it can reduce standby energy consumption.

MRAM can potentially be used in embedded memory and, depending on the implementation and system requirements, as an alternative to some SRAM or DRAM applications.

RRAM

RRAM stores information by changing the electrical resistance of a material. A high-resistance state and low-resistance state can represent different data values.

RRAM is attractive because memory cells can potentially be very small, giving it high-density potential.

Important advantages include:

- Non-volatility

- Simple cell structures

- Potentially high density

- Low-energy operation in suitable implementations

- Potential for 3D integration

- Potential for analog and neuromorphic computing

However, RRAM technology can face challenges involving device variability, resistance-state stability, endurance, and manufacturing consistency.

Access Time

MRAM can provide very fast read access, making it attractive for applications requiring persistent data with relatively low latency.

RRAM can also provide fast access, but actual performance varies considerably with the cell technology, read/write circuitry, array architecture, and operating conditions.

Therefore, neither technology should be judged only by the memory cell itself. System-level access time also depends on controllers, interconnects, sensing circuits, and memory architecture.

Throughput

Both technologies can achieve high throughput when implemented with parallel memory arrays.

MRAM benefits from fast read/write operation and high endurance, while RRAM's high-density arrays provide strong potential for high parallel throughput.

In practice, throughput depends on:

- Memory-array organization

- Number of banks

- Bus width

- Parallelism

- Controller architecture

- Read/write timing

- Technology generation

Energy Efficiency

MRAM has an important energy advantage because it is non-volatile and does not require refresh.

RRAM also has significant potential for low-energy operation because resistance switching can require relatively little energy, although actual energy consumption depends strongly on the switching mechanism and device design.

MRAM vs RRAM Summary

MRAM is particularly attractive when:

- Very high endurance is required.

- Fast access is important.

- Non-volatility is needed.

- Reliable embedded memory is required.

RRAM is particularly attractive when:

- Very high density is required.

- Low power is important.

- 3D integration is desirable.

- New computing architectures such as neuromorphic systems are being considered.

| Technology | Relative Latency | Bandwidth/Throughput | Volatile? | Main Application |
| --- | --- | --- | --- | --- |
| L1 Cache / SRAM | Extremely low | Extremely high | Yes | CPU immediate-access cache |
| L2 Cache / SRAM | Very low | Very high | Yes | Secondary CPU cache |
| L3 Cache / SRAM | Low compared with DRAM | High | Yes | Large shared CPU cache |
| DRAM | Low–moderate | Very high | Yes | Main memory |
| NAND Flash | High | High, especially sequential | No | SSD/storage |
| MRAM | Very low–low | High | No | Embedded/persistent memory |
| RRAM | Low–moderate, device-dependent | High potential | No | High-density/emerging memory |

Code of Conduct:

I GOKUL RAJ 192521494	 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |