SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | Gokul raj | Reg.No.: | 192521494 |
| Department: | IT | Date: | 27 08 26 |

ANNEXURE A

Constraint-Based Problem Solving

- Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

  - CPU utilization must be below 15%

  - Multiple sensors generate interrupts every 1 ms

  - Use vectored interrupt handling

  - Select between memory-mapped and I/O-mapped I/O for optimal latency

I/O Communication and Interrupt Handling Mechanisms

1. I/O Communication Mechanism for a Real-Time Embedded System

Given Constraints

- CPU utilization must be below 15%.

- Multiple sensors generate interrupts every 1 ms.

- Vectored interrupt handling must be used.

- Memory-mapped I/O and I/O-mapped I/O must be evaluated and the option with better latency must be selected.

Proposed I/O Communication Mechanism

A suitable architecture for the real-time embedded system consists of multiple sensors, a vectored interrupt controller, CPU, memory, and a real-time operating system.

Architecture:

Sensors → Interrupt Controller → CPU → Sensor Processing Task

Each sensor is connected to the processor through an I/O interface such as SPI, I²C, UART, or a dedicated interface. Whenever a sensor has new data, it generates an interrupt. The interrupt controller identifies the source and sends the corresponding interrupt vector to the CPU.

Vectored Interrupt Handling

In vectored interrupt handling, every interrupt source is assigned a unique vector address. When an interrupt occurs, the processor directly jumps to the corresponding Interrupt Service Routine (ISR).

For example:

- Vector 1 → Sensor 1 ISR

- Vector 2 → Sensor 2 ISR

- Vector 3 → Sensor 3 ISR

- Vector 4 → Sensor 4 ISR

This reduces the time required to identify the interrupt source and therefore reduces interrupt latency.

The ISR should perform only essential operations:

- Save the required processor context.

- Read or capture the sensor status/data.

- Clear the interrupt request.

- Store the data in a buffer.

- Signal a processing task.

- Return from the ISR.

Complex calculations should be performed outside the ISR.

CPU Utilization

If each sensor generates one interrupt every 1 ms, each sensor generates:

1000 interrupts/second

For Nsensors:

Total Interrupts=1000N

For example, if six sensors are used:

6×1000=6000 interrupts/second

If the ISR requires 10 µs:

CPU Utilization=6000×10μs
=0.06=6%

Therefore:

CPU Utilization=6%<15%

Thus, the CPU utilization requirement is satisfied.

Memory-Mapped I/O vs I/O-Mapped I/O

| Feature | Memory-Mapped I/O | I/O-Mapped I/O |
| --- | --- | --- |
| Address space | Uses memory address space | Uses separate I/O address space |
| Instructions | Normal load/store instructions | Special I/O instructions |
| Hardware integration | Simple | More specialized |
| Addressing flexibility | High | Lower |
| Embedded-system usage | Very common | Less common |
| Latency | Very low | Low |
| Suitability | Highly suitable | Less suitable |

Memory-mapped I/O is selected because it allows device registers to be accessed using normal load/store instructions and integrates efficiently with modern embedded processors. It also provides low-latency register access.

Final Design

The recommended mechanism is:

Sensors → Vectored Interrupt Controller → Short ISRs → Buffers → Real-Time Processing Tasks

Memory-mapped I/O is selected to achieve low access latency, while short ISRs and buffering keep CPU utilization below 15%.

- Design an I/O subsystem under these constraints:

  - High-speed storage requires throughput > 3 GB/s

  - Must use NVMe over PCIe

  - CPU should not handle bulk data transfer

  - Integrate DMA controller with interrupt notification

High-Speed NVMe I/O Subsystem

Given Constraints

- Storage throughput must be greater than 3 GB/s.

- NVMe over PCIe must be used.

- The CPU should not handle bulk data transfer.

- DMA must be integrated with interrupt notification.

Proposed Architecture

A high-speed NVMe subsystem should consist of an NVMe SSD, PCIe interface, system memory, DMA capability in the NVMe controller, CPU, and interrupt controller.

Architecture:

CPU → NVMe Driver → PCIe → NVMe Controller → NAND Flash

↓

DMA

↓

System RAM

NVMe over PCIe

NVMe is specifically designed for high-speed non-volatile storage. It communicates with the host through PCIe and uses submission and completion queues.

A PCIe 4.0 x4 interface is capable of providing sufficient bandwidth for an SSD targeting more than 3 GB/s of storage throughput, assuming the SSD, controller, and workload can sustain the required performance. PCIe 5.0 x4 provides additional bandwidth headroom.

DMA-Based Data Transfer

The CPU should not copy storage data directly. Instead, the NVMe controller transfers data directly between the SSD and system memory using DMA.

The process is:

- The CPU creates an NVMe command.

- The command is placed into a submission queue.

- The NVMe controller reads the command.

- The controller accesses the required NAND Flash data.

- DMA transfers the data directly to system RAM.

- The controller updates the completion queue.

- An MSI/MSI-X interrupt notifies the CPU.

- The CPU processes the completion.

The data path is therefore:

NVMe SSD → NVMe Controller → DMA → RAM

rather than:

NVMe SSD → CPU → RAM

This greatly reduces CPU overhead.

Interrupt Notification

NVMe can use MSI-X interrupts for completion notification. Multiple queues can be associated with different CPU cores.

For example:

- Queue 0 → CPU Core 0

- Queue 1 → CPU Core 1

- Queue 2 → CPU Core 2

- Queue 3 → CPU Core 3

This provides parallel processing and reduces interrupt contention.

Throughput

The target is:

Throughput>3 GB/s

A PCIe 4.0 x4 connection provides sufficient theoretical link capacity to support this target, although actual performance depends on the NVMe SSD, controller, NAND configuration, queue depth, thermal conditions, and workload.

Advantages

- Very high storage bandwidth.

- Low CPU overhead.

- Direct memory transfer using DMA.

- Efficient interrupt notification.

- Multiple queues provide parallelism.

- Suitable for high-performance embedded and computer systems.

Final Design

The recommended architecture is:

CPU → NVMe Driver → PCIe → NVMe Controller → DMA → RAM

The CPU handles commands and completion processing, while the DMA engine handles bulk data transfer. This satisfies the requirement of greater than 3 GB/s throughput while minimizing CPU intervention.

- Construct a multi-device communication architecture with constraints:

  - 6 peripherals using USB, 2 high-speed devices using Thunderbolt

  - Avoid interrupt starvation

  - Use hardware and software interrupts

  - Ensure fair bus arbitration

Multi-Device Communication Architecture

Given Constraints

- Six peripherals use USB.

- Two high-speed devices use Thunderbolt.

- Interrupt starvation must be avoided.

- Hardware and software interrupts must be used.

- Fair bus arbitration must be ensured.

Proposed Architecture

The system consists of a CPU, interrupt controller, USB host controller, USB hub, Thunderbolt controller, DMA engines, and eight peripheral devices.

Architecture:

CPU
↓
Interrupt Controller
↓
USB Host Controller → USB Hub → USB 1 to USB 6
↓
Thunderbolt Controller → Thunderbolt Device 1 and Device 2

USB Devices

Six USB devices can be connected through a USB host controller and one or more USB hubs.

The USB host controller manages USB communication and scheduling. The CPU does not need to continuously poll all six devices.

When a transfer is completed or an important event occurs, the controller generates an interrupt.

Thunderbolt Devices

The two high-speed devices should be connected through the Thunderbolt controller. Since Thunderbolt can carry high-speed PCIe traffic, DMA can be used for large data transfers where supported.

This prevents the CPU from becoming a bottleneck.

Hardware Interrupts

Hardware interrupts are used for time-critical events such as:

- High-priority device events.

- DMA completion.

- Communication errors.

- Data arrival requiring immediate processing.

The process is:

Device → Hardware Interrupt → Interrupt Controller → CPU → ISR

Software Interrupts

Software interrupts are used for deferred processing after the immediate hardware event has been handled.

The process is:

Hardware ISR → Store Event/Data → Software Interrupt → Driver Processing

This keeps hardware ISRs short and prevents one device from occupying the CPU for too long.

Avoiding Interrupt Starvation

Interrupt starvation occurs when a low-priority device is continuously prevented from receiving CPU service because higher-priority interrupts occur repeatedly.

To prevent starvation, the interrupt controller should use:

- Multiple priority levels.

- Interrupt masking.

- Fair scheduling.

- Interrupt moderation where appropriate.

- Bounded ISR execution time.

High-priority devices should receive immediate service, but low-priority devices must also receive service within a guaranteed time.

Fair Bus Arbitration

A round-robin arbitration mechanism can be used to ensure fairness.

For example:

USB1 → USB2 → USB3 → USB4 → USB5 → USB6 → TB1 → TB2 → USB1

Each device gets an opportunity to use the bus.

For high-speed devices, weighted round-robin arbitration can be used. Thunderbolt devices may receive more bus bandwidth because of their higher throughput requirements, while USB devices are still guaranteed service.

Final Design

The system should use:

USB/Thunderbolt Controllers + DMA + Hardware Interrupts + Software Interrupts + Priority Scheduling + Round-Robin/Weighted Arbitration

This architecture provides high performance while preventing interrupt starvation and ensuring fair access to system resources.

- Design a data acquisition system with constraints:

  - Continuous data stream at 500 MB/s

  - Minimal CPU intervention required

  - Must compare DMA transfer vs interrupt-driven I/O

  - Use memory-mapped I/O for device registers

Data Acquisition System with 500 MB/s Continuous Data Stream

Given Constraints

- Continuous data rate = 500 MB/s.

- CPU intervention must be minimal.

- DMA transfer must be compared with interrupt-driven I/O.

- Device registers must use memory-mapped I/O.

Proposed Architecture

A high-speed data acquisition device should be connected to a DMA controller and system memory.

Architecture:

Data Acquisition Device
↓
Memory-Mapped Control Registers
↓
DMA Controller
↓
Circular Buffer in RAM
↓
CPU/Data Processing

Memory-Mapped I/O

The device registers are assigned specific memory addresses.

For example:

- Control Register

- Status Register

- DMA Address Register

- DMA Length Register

- Interrupt Status Register

The CPU accesses these registers using normal load/store instructions.

The CPU initially configures the device and DMA controller through MMIO. After configuration, the DMA controller transfers the incoming data directly to RAM.

DMA Transfer

The DMA mechanism operates as follows:

- CPU configures the DMA destination address.

- CPU specifies the transfer size.

- CPU enables the data acquisition device.

- Device starts producing data.

- DMA transfers data directly from the device to RAM.

- DMA updates the buffer position.

- An interrupt is generated after a buffer or block is completed.

- CPU processes the completed buffer.

The data path is:

Device → DMA → RAM

The CPU does not need to copy each sample.

r

With interrupt-driven I/O, the device generates an interrupt whenever data is ready.

At 500 MB/s, if 4 KB blocks are transferred:

500×1064096≈122,070

interrupts per second would be required.

Such a high interrupt rate would consume significant CPU resources.

Therefore, interrupt-driven I/O is not suitable for this application.

DMA Solution

DMA can transfer large blocks of data without CPU involvement.

For example, if 4 MB DMA buffers are used:

500MB/s4MB=125

buffer-completion interrupts per second are required.

This is much more manageable than approximately 122,000 interrupts per second.

Circular Buffer

A circular buffer can be used to ensure continuous data acquisition.

Buffer A → Buffer B → Buffer C → Buffer D → Buffer A

While DMA fills one buffer, the CPU processes another completed buffer.

This prevents data loss and allows continuous operation.

Final Design

The recommended system is:

Data Acquisition Device → MMIO Control → DMA → Circular RAM Buffer → CPU Processing

DMA is clearly preferred over interrupt-driven I/O because the system must continuously handle 500 MB/s while minimizing CPU intervention.

5.Construct an interrupt handling mechanism with constraints:

  - Support nested interrupts

  - Maximum interrupt latency < 10 µs

Interrupt Handling Mechanism

Given Constraints

- Nested interrupts must be supported.

- Maximum interrupt latency must be less than 10 µs.

- Vectored interrupts must be used for high-priority devices.

- Software interrupts must be used for background tasks.

Proposed Architecture

The interrupt system consists of a CPU, vectored interrupt controller, priority logic, high-priority device interrupts, normal hardware interrupts, and software interrupts.

Architecture:

CPU
↓
Vectored Interrupt Controller
↓
Priority Management
├── High-Priority Hardware Interrupts
├── Normal Hardware Interrupts
└── Software Interrupts

Interrupt Priority

A priority scheme can be defined as follows:

| Priority | Interrupt | Example |
| --- | --- | --- |
| Highest | Critical hardware interrupt | Emergency/time-critical device |
| Very High | High-speed data interrupt | Data acquisition |
| High | Communication interrupt | Network/communication |
| Medium | Normal I/O interrupt | USB/peripheral |
| Low | Non-critical interrupt | Status events |
| Background | Software interrupt | Deferred processing |

Nested Interrupts

Nested interrupts allow a higher-priority interrupt to interrupt a lower-priority ISR.

For example:

- CPU is executing a low-priority ISR.

- A high-priority interrupt occurs.

- CPU saves the current ISR context.

- CPU executes the high-priority ISR.

- High-priority ISR completes.

- CPU restores the previous context.

- Low-priority ISR continues.

The sequence is:

Low-Priority ISR → High-Priority ISR → Return → Low-Priority ISR

This ensures that important real-time events are handled immediately.

Vectored Interrupts

High-priority devices should have dedicated interrupt vectors.

For example:

- Vector 0 → Critical device ISR

- Vector 1 → Data acquisition ISR

- Vector 2 → Communication ISR

- Vector 3 → Storage ISR

- Vector 4 → USB ISR

When an interrupt occurs, the CPU directly accesses the appropriate ISR through the vector table.

This reduces interrupt identification time and helps achieve low latency.

Interrupt Latency

Interrupt latency can be represented as:

Tlatency=Tdetection+Tpriority+Tcontext+Tvector+TISR entry

The required condition is:

Tlatency<10μs

A suitable design target could be:

| Operation | Target Time |
| --- | --- |
| Interrupt detection | 1 µs |
| Priority determination | 1 µs |
| Context saving | 2 µs |
| Vector lookup | 1 µs |
| ISR entry | 1 µs |
| Timing margin | 4 µs |
| Total | 10 µs maximum target |

In an actual implementation, the measured worst-case latency should be kept below 10 µs with sufficient engineering margin rather than designing to exactly 10 µs.

Techniques for Reducing Interrupt Latency

- Use a hardware vectored interrupt controller.

- Assign high priority to time-critical devices.

- Support nested interrupts.

- Keep high-priority ISRs very short.

- Avoid disabling interrupts for long periods.

- Use fast memory for critical interrupt code when supported.

- Avoid unnecessary context saving.

- Use DMA for large data transfers.

- Use interrupt moderation for high-rate devices where real-time requirements permit.

- Move non-critical processing outside the hardware ISR.

Software Interrupts for Background Tasks

Software interrupts are used for operations that do not require immediate hardware response.

For example:

Hardware Interrupt → Short ISR → Software Interrupt → Background Processing

Suitable background tasks include:

- Data processing.

- Buffer management.

- Protocol processing.

- Logging.

- Error reporting.

- Non-critical calculations.

- Driver housekeeping.

This approach prevents long processing operations from blocking high-priority hardware interrupts.

  - Use vectored interrupts for high-priority devices

  - Use software interrupts for background tasks

Code of Conduct:

I GOKUL RAJ 102521449	 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |