# SIMATS ENGINEERING

# ASSESSMENT TOOL 1

COURSE NAME:	COMPUTER ARCHITECTURE	COURSE CODE:	CSA12

COURSE OUTCOME COVERED

CO1: Analyze the functional units of a computer system including CPU, memory, bus structures, and I/O components by examining instruction execution cycles, addressing modes, and performance metrics such as CPI, clock cycles, and benchmarking (BL4) Assessment Tool Weightage: 40%

QUESTIONS:5	Total Marks:25

ANNEXURE A

APPLICATION BASED QUESTIONS

Code of Conduct:

M.GOKUIL RAJ  192521494	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

ANNEXURE A

- A smart city surveillance system processes continuous video feeds from multiple cameras, where large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours, the system experiences lag and delayed response in threat detection. Analyze how the interaction between CPU, memory, and I/O contributes to this issue, and explain how different bus structures (single-bus vs multi-bus) and improvements in instruction execution cycle can enhance system performance.

- Functional Units of a Computer System

  - Central Processing Unit (CPU)

    - Executes instructions and controls system operations.

    - Consists of:

      - Arithmetic Logic Unit (ALU): Performs arithmetic and logical operations.

      - Control Unit (CU): Controls instruction execution and coordinates data transfer.

      - Registers: Store temporary data and instructions for fast access.

Role in Surveillance System:

    - Processes image frames.

    - Runs AI algorithms for object detection and threat identification.

    - Makes real-time decisions.

  - Memory Unit

    - Stores instructions, operating system, and video data.

    - Types:

      - Cache Memory: Fastest memory used for frequently accessed data.

      - RAM: Temporarily stores video frames and processing data.

      - Secondary Storage (SSD/HDD): Stores recorded surveillance footage.

Problem During Peak Hours:

    - Large video streams quickly occupy RAM.

    - Frequent memory accesses increase waiting time for the CPU.

  - Input/Output (I/O) Devices

    - Input: CCTV cameras, motion sensors, microphones.

    - Output: Monitors, alarms, notification systems.

The I/O subsystem transfers huge amounts of video data to memory through system buses.

  - Bus Structure

A bus connects CPU, memory, and I/O devices.

Types of Buses

    - Address Bus

    - Data Bus

    - Control Bus

- Interaction Between CPU, Memory, and I/O Data Flow

CCTV Cameras

↓

I/O Controller

↓

System Bus

↓ Memory

↓ CPU

↓

Threat Detection

↓

Alarm / Display

- Single-Bus vs Multi-Bus Structure

| Feature | Single Bus | Multi-Bus |
| --- | --- | --- |
| Number of buses | One | Multiple |
| Data transfer | One transfer at a time | Multiple simultaneous transfers |
| CPU waiting time | High | Low |
| Bus congestion | High | Low |
| Cost | Low | Higher |
| Performance | Moderate | High |

# Single-Bus Architecture

CPU

|

|	System Bus

|	|	|

Memory	I/O	Storage

# Multi-Bus Architecture

CPU

/	\

Memory Bus	I/O Bus

|	|

Memory	Cameras

# Instruction Execution Cycle

The CPU executes each instruction in four stages.

# Step 1: Fetch

- Retrieve instruction from memory.

# Step 2: Decode

- Control Unit interprets the instruction.

# Step 3: Execute

- ALU performs computation.

# Step 4: Store (Write Back)

- Store the result in registers or memory.

# Improvements in Instruction Execution Cycle

  - Instruction Pipelining

    - Executes multiple instruction stages simultaneously.

    - Increases CPU utilization.

    - Reduces processing time.

# Example:

| Clock | Instruction 1 | Instruction 2 | Instruction 3 |
| --- | --- | --- | --- |
| 1 | Fetch |  |  |
| 2 | Decode | Fetch |  |
| 3 | Execute | Decode | Fetch |
| 4 | Store | Execute | Decode |

# Superscalar Processing

    - Executes multiple instructions in one clock cycle. Benefit:

    - Faster image processing.

- A real-time stock trading application running on a processor executes millions of instructions per second, but users report delays in transaction processing during high load. The system has a high CPI due to frequent memory access and branch instructions. Examine how the fetch–decode–execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and improve overall CPU performance using suitable architectural enhancements.

# Analysis of a Real-Time Stock Trading Application

A real-time stock trading application executes millions of instructions every second to process market data, execute buy/sell orders, and update user accounts. During periods of high market activity, users experience delays because the processor has a high Cycles Per Instruction (CPI) caused by frequent memory accesses and branch instructions. These factors increase CPU execution time and reduce overall system performance.

# Fetch–Decode–Execute Cycle

The CPU processes every instruction through three main stages.

# Step 1: Fetch

  - The CPU fetches the next instruction from memory.

  - The Program Counter (PC) stores the address of the next instruction.

# Issue:

  - Frequent memory accesses increase fetch time.

  - Cache misses force the CPU to wait for slower main memory.

# Step 2: Decode

  - The Control Unit interprets the instruction.

  - Determines the required operation and operands.

# Issue:

  - Complex branch instructions require additional decoding.

  - Incorrect branch predictions waste CPU cycles.

# Step 3: Execute

  - The ALU performs arithmetic or logical operations.

  - Results are written back to registers or memory.

# Issue:

  - Memory-intensive instructions delay execution.

  - CPU stalls while waiting for data from memory.

# Instruction Cycle

Fetch

↓ Decode

↓ Execute

↓

Write Back

↓

Next Instruction

# Causes of High CPI

  - Frequent Memory Access

    - Large financial datasets are stored in RAM.

    - Cache misses require slow memory access.

    - CPU waits during data retrieval.

# Branch Instructions

Examples:

    - Buy/Sell decision

    - Risk verification

    - Price comparison

    - Fraud detection

Frequent conditional branches create pipeline stalls and increase CPI.

# Cache Misses

    - Requested data is unavailable in cache.

    - CPU accesses slower main memory.

    - Execution time increases.

- An embedded system based on an 8085 microprocessor is used in an industrial temperature control unit, where sensors provide input and actuators respond accordingly. However, incorrect temperature readings are occasionally processed due to improper use of addressing modes in the program. Discuss how different addressing modes influence data access, identify possible causes of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure accurate and reliable operation.

# Analysis of an 8085-Based Industrial Temperature Control Unit

An industrial temperature control unit uses an 8085 microprocessor to read temperature data from sensors, process the values, and control actuators such as heaters or cooling fans.

Incorrect temperature readings may occur due to improper use of addressing modes, causing the processor to access incorrect memory locations or data. Proper use of the 8085 instruction set ensures accurate sensor data processing and reliable system operation.

# Working of the Embedded Temperature Control System

Temperature Sensor

│

▼

Input Interface

│

▼

8085 Microprocessor

│

▼

Data Processing

│

▼

Decision Making

│

▼

Heater / Fan (Actuator)

# . Addressing Modes of 8085

Addressing modes determine how the processor accesses operands during instruction execution.

a) Immediate Addressing Mode

The data is directly included in the instruction.

# Influence of Addressing Modes on Data Access

|  | Addressing Mode | Data Access | Effect |  |
| --- | --- | --- | --- | --- |
|  | Immediate | Data inside instruction | Fast execution |  |
|  | Register | Data in registers | Very fast |  |
|  | Direct | Fixed memory address | Suitable for constant data |  |
|  | Register Indirect | Memory through register pair | Flexible for sensor arrays |  |
|  | Implied | Operand implied | No explicit operand required |  |

# Example Program Flow

Start

│

Read Temperature Sensor

│

Store Reading

│

Compare with Threshold

│

┌──────────────┐

│Temperature > Limit?│

└───────┬──────┘

│Yes

▼

Turn ON Cooling Fan

│

▼

No

│

Turn ON Heater

│

▼ Repeat

# Methods to Ensure Accurate and Reliable Operation

- Use the correct addressing mode for each type of data access.

- Initialize registers and memory before processing.

- Verify register pair values before indirect memory access.

- Store calibration values at fixed memory locations.

- Validate sensor readings using comparison instructions (CMP).

- Avoid overwriting registers containing sensor data.

- Use conditional branching (JZ, JC, JNZ) for accurate decision making.

- Test and debug the program thoroughly to eliminate addressing errors.

# Advantages of Proper Addressing Modes

- Faster program execution.

- Accurate sensor data acquisition.

- Reduced programming errors.

- Reliable temperature monitoring.

- Improved actuator control.

- Better industrial system safety.

- A computing system uses an 8086 microprocessor with segmented memory for executing multiple programs simultaneously, but it encounters issues like incorrect data access and stack overflow errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how improper segment handling can lead to such problems, and explain how instruction execution and addressing modes must be managed to ensure correct program execution.

# Evaluation of 8086 Segmented Memory and Program Execution

An 8086 microprocessor uses segmented memory architecture to efficiently organize memory and execute multiple programs. Memory is divided into Code Segment (CS), Data Segment (DS), Stack Segment (SS), and Extra Segment (ES). If segment registers or addressing modes are used incorrectly, problems such as incorrect data access, stack overflow, and program crashes can occur.

# Segmented Memory Architecture of 8086

The 8086 has a 20-bit address bus, allowing it to access 1 MB of memory. Since its registers are only 16 bits, it uses segmentation to generate the physical address.

Physical Address Formula

Physical Address = (Segment Address × 𝟏𝟎𝑯) + Offset Address

# Example:

  - Segment = 2000H

  - Offset = 1200H

Physical Address = 20000𝐻 + 1200𝐻 = 21200𝐻

- Types of Memory Segments

| Segment | Register | Purpose |
| --- | --- | --- |
| Code Segment | CS | Stores executable instructions |
| Data Segment | DS | Stores variables and data |
| Stack Segment | SS | Stores return addresses and local data |
| Extra Segment | ES | Stores additional data for string operations |

- Working of Segmentation Code Segment (CS)

Stores program instructions. Example:

MOV AX, BX ADD AX, CX

The Instruction Pointer (IP) contains the offset of the next instruction. Physical Address:

𝐶𝑆 × 10𝐻 + 𝐼𝑃

Data Segment (DS)

Stores variables and program data. Example:

MOV AX, [1000H]

The processor accesses memory through the DS register.

Stack Segment (SS) Used for:

- Function calls

- Interrupt handling

- Temporary storage

- Local variables Instructions:

PUSH AX

POP AX CALL RET

Stack Pointer (SP) always points to the top of the stack.

Extra Segment (ES)

Primarily used in string operations. Example:

MOVSB MOVSW

- Problems Due to Improper Segment Handling

  - Incorrect Data Access

If DS points to the wrong segment, Example:

Instead of DS = 3000H

Program accidentally loads DS = 4000H

Result:

    - Wrong memory location accessed

    - Incorrect data processing

    - Invalid program output

  - Stack Overflow

Occurs when excessive PUSH instructions fill the stack beyond its allocated space. Example:

PUSH AX PUSH BX PUSH CX

...

without corresponding POP instructions. Effects:

    - Memory corruption

    - Program crash

    - Loss of return addresses

  - Incorrect Code Execution If CS changes accidentally,

The processor executes unintended instructions. Result:

    - Unexpected behavior

    - System failure

  - Wrong Offset Address

Incorrect offset values cause access to invalid memory locations.

# Stack Memory Operation

# Addressing Modes in 8086

| Addressing Mode | Description | Example |
| --- | --- | --- |
| Immediate | Operand is part of the instruction | MOV AX,1000H |
| Register | Operand in register | MOV AX,BX |
| Direct | Memory address given directly | MOV AX,[2000H] |
| Register Indirect | Address stored in register | MOV AX,[BX] |
| Based | Base register used | MOV AX,[BX+10] |
| Indexed | Index register used | MOV AX,[SI] |
| Based Indexed | Base + Index | MOV AX,[BX+SI] |
| Based Indexed with Displacement | Base + Index + Offset | MOV AX,[BX+SI+20] |

# Ensuring Correct Program Execution

- Initialize CS, DS, SS, and ES correctly before execution.

- Use the appropriate addressing mode for each memory access.

- Balance every PUSH with a corresponding POP instruction.

- Avoid modifying segment registers unintentionally.

- Validate memory addresses before accessing data.

- Allocate sufficient stack space to prevent overflow.

- Use CALL and RET instructions correctly for subroutine handling.

- Test and debug programs to identify segmentation and addressing errors.

# Advantages of Proper Segment Management

  - Efficient utilization of the 1 MB memory space.

  - Faster execution through organized code and data storage.

  - Prevention of stack overflow and memory corruption.

  - Accurate data access and processing.

  - Improved reliability for multitasking applications.

  - Simplified debugging and maintenance.

- A data communication system represents packet information in hexadecimal format for ease of debugging and transmission, but a software engineer mistakenly interprets values, leading to incorrect data processing. Analyze the importance of number system conversions between hexadecimal and binary in such systems, explain how errors in conversion can affect instruction execution, and discuss how efficient CPU design and benchmarking can help detect and minimize such issues in real-time systems.

- Analysis of Number System Conversion in a Data Communication System

A data communication system represents packet information in hexadecimal (Hex) format because it is compact, easy to read, and closely corresponds to binary data used by computers. However, if a software engineer incorrectly converts hexadecimal values to binary, packet information may be interpreted incorrectly, leading to transmission errors, incorrect instruction execution, and system failures. Efficient CPU design and benchmarking help detect and minimize such issues in real-time communication systems.

# Number Systems Used in Data Communication

| Number System | Base | Digits Used |
| --- | --- | --- |
| Binary | 2 | 0, 1 |
| Decimal | 10 | 0–9 |
| Hexadecimal | 16 | 0–9, A–F |

# Importance of Hexadecimal to Binary Conversion

| Hex | Binary |
| --- | --- |
| 0 | 0000 |
| 1 | 0001 |
| 2 | 0010 |
| 3 | 0011 |
| 4 | 0100 |
| 5 | 0101 |
| 6 | 0110 |
| 7 | 0111 |
| 8 | 1000 |
| 9 | 1001 |
| A | 1010 |
| B | 1011 |
| C | 1100 |
| D | 1101 |
| E | 1110 |
| F | 1111 |

# Packet Representation

A communication packet typically consists of:

    - Header

    - Source Address

    - Destination Address

    - Payload

    - Checksum

Hexadecimal values are converted into binary before processing by the CPU.

# Importance of Number System Conversion

- Easy Debugging

  - Hexadecimal values are shorter and easier to read than binary. Example:

Binary : 1101011010011110 Hex	: D69E

- Memory Address Repres

Example:

Memory Address = 1F2AH instead of 0001111100101010₂

# Packet Transmission

Communication protocols (Ethernet, TCP/IP, CAN) commonly display packet contents in hexadecimal format.

# Instruction Representation

Machine instructions are stored in binary but displayed in hexadecimal for easier analysis.

# Errors Due to Incorrect Conversion

- Incorrect Data Interpretation

Suppose Hex = 2F

Correct Binary 00101111

If converted incorrectly as 00111111

The received value changes completely.

# Wrong Instruction Execution

Machine instructions are binary.

Incorrect conversion may generate an invalid instruction. Result:

  - Incorrect execution

  - Program crash

  - Unexpected output

# Incorrect Memory Address

Example Correct Address 4A20H

Incorrectly interpreted as 4B20H

CPU accesses the wrong memory location.

# Packet Corruption

Wrong binary conversion changes:

  - Packet header

  - Payload

  - Checksum Result:

  - Transmission failure

  - Packet rejection

# Instruction Execution Process

The CPU executes instructions through the following stages:

# Step 1 – Fetch

    - Read instruction from memory.

↓

# Step 2 – Decode

    - Convert binary instruction into CPU operations.

↓

# Step 3 – Execute

    - Perform arithmetic or logical operation.

↓

# Step 4 – Store

    - Write result to registers or memory.

Incorrect binary conversion affects the decode stage, causing improper instruction execution.

# Efficient CPU Design to Reduce Errors

- Error Detection

CPUs use:

  - Parity checking

  - Error Correcting Code (ECC)

  - Checksum verification to identify incorrect data.

# Pipeline Verification

Modern processors validate instruction integrity before execution.

# Cache Memory

Frequently used instructions remain in cache, reducing repeated conversions and memory access delays.

# High-Speed ALU

Performs binary operations accurately and efficiently.

# Memory Protection

Prevents unauthorized access to invalid memory locations.

# Benchmarking

Benchmarking measures system performance and reliability. Common metrics include:

| Metric | Purpose |
| --- | --- |
| CPI (Cycles Per Instruction) | CPU efficiency |
| Clock Rate | Processing speed |
| Throughput | Packets processed per second |
| Latency | Time taken to process a packet |
| Error Rate | Number of packet conversion errors |
| Memory Bandwidth | Data transfer speed |

  - Methods to Minimize Conversion Errors

    - Use reliable hexadecimal-to-binary conversion algorithms.

    - Validate packet headers and checksums before processing.

    - Implement parity and ECC for error detection and correction.

    - Use standardized packet formats and communication protocols.

    - Verify memory addresses before execution.

    - Employ CPU benchmarking to identify bottlenecks.

    - Test software thoroughly using debugging tools and simulation.

Advantages of Efficient CPU Design

    - Faster instruction execution.

    - Accurate packet decoding.

    - Reduced communication errors.

    - Improved real-time performance.

    - Increased system reliability.

MARKS ALLOCATION

| Criteria | Understanding of Problem Context (20%) | Application of Concepts (20%) | Problem-Solving Approach (20%) | Accuracy of Solution (20%) | Clarity (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understan ding of Problem Context | 20 | Clearly understands the problem and accurately identifies all requirements | Good understanding with minor gaps | Basic understanding; some requirements unclear | Poor understanding or misinterpretation of the problem |
| Application of Concepts | 30 | Accurately applies appropriate concepts to solve complex problems | Applies relevant concepts correctly with minor errors | Applies basic concepts with limited accuracy | Fails to apply appropriate concepts |
| Problem-Solving Approach | 20 | Logical, well-structured, and efficient approach | Mostly logical approach with minor gaps | Basic approach with some inconsistencies | Illogical or unclear approach |
| Accuracy of Solution | 20 | Completely correct solution with proper justification | Mostly correct with minor errors | Partially correct solution | Incorrect or incomplete solution |
| Clarity | 10 | Clear, well- organized, and properly explained solution | Understandable with minor clarity issues | Limited clarity and explanation | Poorly presented and difficult to understand |

# THANK YOU